SWaT Digital Twin

ML Strategy, Baseline Design & Physical System Report

Cyber-Physical Security Research  |  March 2026

# 1. ML Strategy — Baseline, Variations & Classification

## 1.1  Why You Need a Baseline

A baseline is a model trained exclusively on NORMAL operation data (ATTACK_ID=0). It defines the statistical manifold of healthy plant behaviour. Any departure from this manifold is a candidate anomaly. Without a strong baseline you cannot distinguish a genuine attack from normal process variation.

The baseline serves three roles:

- Threshold calibration — sets the false-positive rate you are willing to accept in production.

- Feature selection — identifies which of the 84 columns carry discriminative information vs. which are pure noise.

- Comparison anchor — every supervised classifier is benchmarked against it. A classifier that barely beats the baseline is not useful.

## 1.2  Baseline Variations You Must Generate

Each run you collect should deliberately vary ONE operational parameter to test whether your ML model learns invariant features (robust) or memorises run-specific artefacts (brittle).

| Variation | What Changes | Register Affected | Why It Matters for ML |
| --- | --- | --- | --- |
| Normal baseline | Nothing — steady state | All stable | Defines normal manifold |
| High turbidity | Raw water turbidity elevated (AIT_201 ~800) | AIT_201, P_206, Coag tank | Tests if model confuses coagulant response with attack |
| Low chlorine demand | Source water low in organics | Chlorine_Residual, P_205 | Model must not flag low-Cl normal as attack |
| UF backwash cycle | Scheduled backwash (UF_Last_BW > 18000) | DPIT_301, MV_303/304, P_602 | Backwash looks like a membrane attack — must not false-positive |
| RO CIP cleaning | Scheduled RO clean (RO_Fouling > 80) | RO_Fouling, P_501=OFF | CIP looks like RO shutdown attack |
| Pump maintenance mode | P_102 / P_502 reserved pumps activated briefly | P_102, P_502, FIT_101 | Unusual pump pattern should not trigger alarm |
| Seasonal temperature | Ambient_Temp changed (20°C vs 35°C) | Water_Temp, viscosity-linked flows | Temperature affects flow and pH drift — must not cause spurious alarms |

## 1.3  Attack Categories for Classification

Your classification target is ATTACK_NAME (or ATTACK_ID). These are the attack types your system must learn to detect:

| Attack | MITRE ID | Registers Affected | Detection Signature |
| --- | --- | --- | --- |
| Reconnaissance | T0840 | All (read-only scan) | High Modbus read frequency from unusual source IP; no write activity |
| Replay Attack | T0843 | All registers frozen | Near-zero variance across all sensors over ≥10 s window |
| pH Manipulation | T0831 | AIT_202, P_203, Acid | pH drops exponentially while acid pump OFF — physical inconsistency |
| Slow Ramp | T0836 | LIT_101, FIT_101 | Tank level drifts 0.2–0.5 L/cycle for >5 min without pump state change |
| Pump Failure Spoof | T0816 | P_101, FIT_101 | P_101=ON but FIT_101 near zero — flow-pump inconsistency |
| Valve Manipulation | T0849 | MV_301–304, FIT_301 | Valve state mismatch with upstream flow; DPIT_301 diverges |
| Multi-stage Stealth | T0830 | pH + flow + TMP + acid | No single variable alarms but Mahalanobis distance from normal manifold spikes |
| Sensor Spoofing | T0856 | Any sensor register | Physical model prediction vs. register value diverges beyond 3σ |

## 1.4  Feature Engineering Pipeline

Raw register values alone are insufficient. Engineer these feature groups before training:

| Feature Group | Examples | Purpose |
| --- | --- | --- |
| Rate of change | d(LIT_101)/dt, d(AIT_202)/dt | Detects unphysical change speed — key for slow ramp and pH attacks |
| Rolling statistics | std(AIT_202, 30), mean(FIT_101, 10) | Captures sustained drift; robust to single-sample noise |
| Physical mass balance | FIT_101 − FIT_201, FIT_301 − FIT_401 | Direct physics violation — cannot be spoofed without matching multiple registers |
| Pump–flow consistency | P_101 AND FIT_101 < 0.5 | Boolean physical constraint — pump ON with no flow is always anomalous |
| pH–pump consistency | AIT_202 > 750 AND P_203 = 0 | Core pH attack signature from ST control logic |
| Fouling acceleration | d²(DPIT_301)/dt² | Second derivative catches exponential fouling attacks before threshold |
| Temporal lag features | LIT_101_t-5, pH_t-10, pH_t-30 | Gives tree-based models temporal context without needing LSTM |
| Pump duty cycle | sum(P_101, 60s) / 60 | Detects intermittent covert manipulation |
| Multivariate distance | Mahalanobis distance from normal mean/cov | Detects multi-stage stealth attacks invisible to univariate rules |

## 1.5  Recommended Model Stack

Use a three-layer ensemble. Train each layer independently to prevent correlation:

| Layer | Model | Training Data | Output | Threshold |
| --- | --- | --- | --- | --- |
| 1 — Pre-filter | Isolation Forest | Normal only | Anomaly score 0–1 | score > 0.6 → send to Layer 2 |
| 2 — Classifier | XGBoost | All labelled runs | Attack class + prob | prob > 0.7 → confirm alarm |
| 3 — Temporal | LSTM (64 units, seq=30) | All labelled runs | Binary + attack type | Applied to flagged windows only |
| Explainer | SHAP values | N/A | Feature importance | Top-3 features per alarm |

## 1.6  Step-by-Step TODO for ML

Phase 1 — Data Collection (Weeks 1–2)

- Run start_system.py for each attack combination using the 5-run structure already set up.

- Collect at minimum: 2× normal runs (different operational conditions), 1× UF backwash-heavy run, 1× multi-attack run, 1× single-attack-per-type run.

- Target: ≥50,000 rows total, ≥40% normal, attack types balanced within the 60%.

- Validate each CSV: check ATTACK_ID distribution, null count, and value ranges using the analysis script.

Phase 2 — Feature Engineering (Week 3)

- Merge all CSVs, sort by Timestamp, reset index.

- Compute all 9 feature groups from section 1.4 above.

- Drop near-zero-variance columns (std < 0.01 across entire dataset).

- Normalise continuous features with RobustScaler (median/IQR — resistant to attack outliers).

- Split: 70% train, 15% validation, 15% test — stratified by ATTACK_ID.

Phase 3 — Baseline & Anomaly Detection (Week 4)

- Train Isolation Forest on normal rows only (contamination=0.05).

- Tune threshold on validation set to achieve FPR < 3%.

- Train Autoencoder on normal rows only (encoder: 128→64→32, decoder: 32→64→128).

- Record baseline metrics: precision, recall, F1, confusion matrix per attack type.

Phase 4 — Supervised Classification (Weeks 5–6)

- Train XGBoost with scale_pos_weight = n_normal / n_attack.

- Hyperparameter search: max_depth 4–10, learning_rate 0.01–0.1, n_estimators 200–500.

- Evaluate on test set. Target: F1 > 0.88 per attack class.

- Train LSTM on 30-step windows. Apply only to windows flagged by XGBoost.

- SHAP analysis: identify top-5 features per attack type for operator explainability.

Phase 5 — Evaluation & Robustness (Weeks 7–8)

- Test against unseen run (held-out dataset from a different day).

- Evaluate transfer: does a model trained on run_01–04 generalise to run_05?

- Adversarial test: manually inject a slow attack below detection threshold — does LSTM catch it?

- Write up confusion matrices, ROC curves, and SHAP plots.

# 2. Physical Cyber-Physical System — Requirements & Hardware

## 2.1  System Architecture Overview

The physical SWaT testbed replicates a 6-stage water treatment process with real sensors, actuators, and a PLC network. The architecture has three layers: field layer (sensors/actuators), control layer (PLCs), and supervisory layer (SCADA/historian).

## 2.2  PLC — Siemens SIMATIC S7-1200

The S7-1200 is the industry-standard entry-level PLC used in the original iTrust SWaT testbed at Singapore. It runs CODESYS-compatible Structured Text and supports Modbus TCP natively.

| Model | Specification | Link / Part Number |
| --- | --- | --- |
| CPU 1214C DC/DC/DC | 14 DI / 10 DO / 2 AI — onboard; 100 kHz HSC; PROFINET | Part: 6ES7214-1AG40-0XB0 Siemens TIA Portal |
| SB 1232 AO | 1× analog output 0–20 mA / 0–10V for valve positioner | Part: 6ES7232-4HA30-0XB0 |
| SM 1231 AI 8× | 8× analog inputs 4–20 mA for pH/ORP/flow/level sensors | Part: 6ES7231-5PF32-0XB0 |
| SM 1222 DO 16× | 16× digital outputs 24V DC for pump relays and solenoid valves | Part: 6ES7222-1BH32-0XB0 |
| CM 1241 RS485 | Serial comms module for Modbus RTU legacy instruments | Part: 6ES7241-1CH32-0XB0 |

Alternative budget PLC: Allen-Bradley MicroLogix 1100 or Click Plus PLC (Automation Direct C2-03CPU) at ~$200 USD for a proof-of-concept build.

## 2.3  Sensors Required per Stage

| Stage | Instrument | Measurement | Signal | Model | Approx. Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| S1 | LIT-101 | Tank level | 4–20 mA | Endress+Hauser FMU40 | ~$350 |
| S1 | FIT-101 | Inlet volumetric flow | 4–20 mA | Keyence FD-Q series | ~$400 |
| S2 | AIT-202 (pH) | pH 0–14 | 4–20 mA | Hach PHD sc | ~$800 |
| S2 | AIT-201 (Turb) | Turbidity NTU | 4–20 mA | Hach TU5400sc | ~$1200 |
| S2 | AIT-203 (ORP) | ORP mV | 4–20 mA | Endress+Hauser CPS16D | ~$600 |
| S3 | DPIT-301 | Differential pressure TMP | 4–20 mA | Endress+Hauser PMD75 | ~$700 |
| S3 | LIT-301 | UF permeate tank level | 4–20 mA | Gems XT-900 | ~$300 |
| S4 | AIT-402 (ORP) | Post-dechlor ORP | 4–20 mA | Hach CLF10 sc | ~$900 |
| S5 | PIT-501 | RO feed pressure 0–25 bar | 4–20 mA | Endress+Hauser PMP71 | ~$500 |
| S5 | AIT-501 (TDS) | TDS conductivity | 4–20 mA | Hach COND3500 | ~$650 |
| S6 | FIT-601 | Distribution flow | 4–20 mA | Krohne OPTIFLUX 2000 | ~$1100 |

## 2.4  Actuators Required

| Tag | Type | Specification | Model | Notes |
| --- | --- | --- | --- | --- |
| MV-101 | Motorised ball valve | DN25, 24V DC, 4–20 mA | Bürkert 2830 | Inlet control — open/close + position feedback |
| P-101/102 | Centrifugal pump | 1/4 HP, 12 L/min, 24V DC | Procon rotary vane pump | Main and booster feed — relay controlled |
| P-203/205/206 | Peristaltic dosing pump | 0–100 mL/min, 4–20 mA | Prominent BETA 4 | Chemical dosing — analogue speed control |
| UF Membrane | Hollow-fibre UF module | 0.01 µm, 10 L/hr | Koch Puron 5-inch | Can substitute with lab-scale membrane for POC |
| RO Membrane | Spiral wound RO element | 2540 format, 75 GPD | Filmtec TW30-2540 | Standard residential RO element |
| UV-401 | UV disinfection unit | 40 mJ/cm², 10 L/min | Trojan UV Swift 6 | Relay ON/OFF only |
| P-501 | High-pressure RO pump | 150 PSI, 12 V DC | Aquatec CDP 8800 | Pressure booster for RO stage |

## 2.5  Network Architecture

The physical testbed uses an OT network architecture with deliberate air-gap segmentation to enable realistic attack scenarios:

| Layer | Protocol / Device | Description |
| --- | --- | --- |
| Field (Level 0) | 4–20 mA / HART | All sensors transmit 4–20 mA analogue signal to PLC AI modules. HART overlaid for diagnostics. |
| Control (Level 1) | Modbus TCP / PROFINET | S7-1200 PLC runs control logic. Communicates with SCADA over Ethernet/IP on isolated VLAN. |
| Supervisory (Level 2) | OPC-UA / Modbus TCP | SCADA historian (e.g. FactoryTalk or open-source OpenPLC Runtime). Python bridge connects here. |
| DMZ (Level 3) | REST / WebSocket | Logging, ML inference server, and dashboard. Physically separated from OT network. |
| Attack surface | Modbus TCP port 502/1502 | Intentionally exposed on VLAN for controlled attack injection via Python scripts. |

## 2.6  Estimated Physical Build Cost

| Component | Qty | Unit Cost (USD) | Subtotal |
| --- | --- | --- | --- |
| S7-1214C CPU + power supply | 1 | $420 | $420 |
| SM 1231 AI 8× analog input module | 2 | $280 | $560 |
| SM 1222 DO 16× digital output module | 1 | $240 | $240 |
| pH + ORP sensors (AIT-202, AIT-203, AIT-402) | 3 | $600 | $1800 |
| Flow meters (FIT-101, 201, 301, 401, 501, 601) | 6 | $350 | $2100 |
| Level transmitters (LIT-101, 301, 401) | 3 | $300 | $900 |
| Turbidity sensor (AIT-201) | 1 | $1200 | $1200 |
| Pressure transmitters (PIT-501, 502, DPIT-301) | 3 | $500 | $1500 |
| Pumps (centrifugal + dosing + HP) | 8 | $200 | $1600 |
| Motorised valves (MV-101, 201, 301–304) | 6 | $150 | $900 |
| UF membrane + housing | 1 | $600 | $600 |
| RO membrane + pressure vessel | 1 | $300 | $300 |
| UV unit, tanks, pipework, DIN rail, wiring | — | Lump sum | ~$1500 |
| TOTAL ESTIMATE |  |  | ~$13,620 USD |

Note: Academic lab pricing with vendor discounts can reduce this by 20–30%. The Singapore iTrust SWaT testbed cost approximately $1M USD at full industrial scale with 6 stages, redundant PLCs, and Wonderware SCADA.

## 2.7  Key Reference Links

iTrust SWaT Dataset (NUS): https://itrust.sutd.edu.sg/itrust-labs_datasets/dataset_info/

Siemens S7-1200 Product Page: https://www.siemens.com/global/en/products/automation/systems/industrial/plc/s7-1200.html

OpenPLC Runtime (free CODESYS-compatible): https://autonomylogic.com/

Filmtec TW30-2540 RO Membrane: https://www.dupont.com/water/resources/filmtec-membranes.html

pymodbus library: https://pymodbus.readthedocs.io/

MITRE ATT&CK for ICS: https://attack.mitre.org/matrices/ics/

# 3. 2-Month Research Roadmap — What Is Feasible

## 3.1  Already Completed (as of March 2026)

| Deliverable | Status | Evidence |
| --- | --- | --- |
| MATLAB physics simulation (all 6 stages) | ✅ Done | swat_physics_server.m — TCP server with ODE, pH kinetics, fouling, RO |
| CODESYS ST control logic (all 6 stages) | ✅ Done | plant.st — pump/valve logic, alarms, interlocks, pH safety |
| Python Modbus bridge (physics↔PLC) | ✅ Done | physics_client.py — 10 Hz loop, TCP to MATLAB, Modbus to CODESYS |
| Dataset generator with 7 attack types | ✅ Done | automated_dataset_generator.py — attack scheduler, exponential profiles |
| Live P&ID dashboard (WebSocket) | ✅ Done | dashboard.html — all 6 stages, pump/valve symbols, alarm overlays |
| First dataset run collected (run_01) | ✅ Done | 9152 rows, 84 columns, 2 attack types (cut short — need full 70 min run) |
| Mathematical analysis documentation | ✅ Done | Sigmoid, exponential, Gaussian — all physics functions justified |

## 3.2  Month 1 Objectives (April 2026)

| Wk | Objective | Deliverable | Feasibility | Notes |
| --- | --- | --- | --- | --- |
| 1 | Complete 5 full dataset runs (70 min each, 5 attack combos) | run_01 – run_05 CSVs | ✅ High | Fix PIT_502 threshold in ST first |
| 1 | Feature engineering pipeline (all 9 feature groups) | feature_engineer.py | ✅ High | ~2 days coding |
| 2 | Baseline Isolation Forest — train on normal data only | baseline_model.pkl | ✅ High | sklearn 5-minute train |
| 2 | Baseline Autoencoder — train on normal data only | autoencoder.h5 | ✅ High | Keras/TF, ~30 min train |
| 2 | Threshold calibration — FPR < 3% on validation normal data | threshold_report.pdf | ✅ High | Critical before supervised training |
| 3 | XGBoost multiclass classifier — all 7 attack types | xgb_classifier.pkl | ✅ High | GPU not needed |
| 3 | SHAP analysis — feature importance per attack type | shap_plots.png | ✅ High | pip install shap |
| 4 | LSTM on 30-step windows for temporal attack detection | lstm_model.h5 | ⚠ Medium | Needs GPU or 2–4 hr CPU train |
| 4 | Confusion matrix + ROC curves — all models vs all attack types | evaluation_report.md | ✅ High | Target F1 > 0.88 per class |

## 3.3  Month 2 Objectives (May 2026)

| Wk | Objective | Deliverable | Feasibility | Notes |
| --- | --- | --- | --- | --- |
| 5 | Adversarial testing — inject attacks below detection threshold | adversarial_results.csv | ✅ High | Modify attack amplitude in generator |
| 5 | Transfer test — model trained on runs 1–4 vs held-out run 5 | transfer_eval.md | ✅ High | Key generalisation metric |
| 6 | Real-time inference pipeline — integrate XGBoost into ws_server.py | live_detector.py | ⚠ Medium | Adds ~1 ms latency per cycle |
| 6 | Dashboard alarm integration — ML model fires alarm in real time | dashboard_v2.html | ✅ High | Add WS message field for ML_ALARM |
| 7 | POC physical component: single S7-1200 + 2 sensors + 1 pump | Physical rig photos | ⚠ Medium | $800–1200 for minimal POC |
| 7 | Dataset variation run: UF backwash-heavy normal baseline | run_06 CSV | ✅ High | Tests FPR on maintenance events |
| 8 | Final paper / technical report writing | report_draft.docx | ✅ High | This document is the foundation |
| 8 | GitHub repository — clean code, README, reproducibility instructions | github.com/... repo | ✅ High | Makes research reproducible |

## 3.4  What Is NOT Feasible in 2 Months

| Item | Status | Reason |
| --- | --- | --- |
| Full 6-stage physical build | ❌ Not feasible | $13K+ cost + 3–4 months procurement + lab space required |
| Industrial SCADA (FactoryTalk, Wonderware) | ❌ Not feasible | License cost $10K+, 6-week implementation |
| Multi-PLC networked testbed (6 PLCs) | ❌ Not feasible | Requires dedicated network lab, $5K+ hardware |
| Deep learning architecture search (NAS) | ⚠ Stretch goal | Requires GPU cluster; feasible on Google Colab T4 but time-intensive |
| Federated learning across multiple PLCs | ⚠ Stretch goal | Requires physical multi-node setup; can simulate on single machine |

# 4. Physical System Schematic Description

The diagram below describes the physical layout. Full SVG/CAD drawing available in the dashboard HTML file (swat_dashboard.html) generated alongside this report.

### Process Flow (Physical)

Raw water enters T-101 (100L HDPE tank) via a solenoid-controlled inlet. P-101/P-102 transfer to the dosing stage where chemical pumps (P-203 acid, P-205 chlorine, P-206 coagulant) inject reagents monitored by pH (AIT-202), turbidity (AIT-201) and ORP (AIT-203) sensors. Flow passes through T-301 UF housing with a hollow-fibre membrane (DPIT-301 monitors TMP). P-401 transfers permeate to T-401 (UV-401 installed in-line, P-403 bisulfate dosing for Cl removal). P-501 pressurises feed through the RO element (PIT-501 feed pressure, PIT-502 permeate pressure). Permeate flows to distribution header via P-601/P-603 (FIT-601). Brine exits to drain.

### Network Diagram

S7-1200 CPU on OT VLAN (192.168.5.0/24) → Modbus TCP port 1502 → Python bridge PC (192.168.5.100) → MATLAB physics server TCP port 9501 (localhost) → ws_server.py WebSocket port 8765 → Dashboard browser. Attack injection PC on same OT VLAN (192.168.5.200) with Python scripts accessing port 1502 directly.

End of Report.